- The C2 platform cannot talk directly to “a drone” in the abstract. It needs a physical communications path, a protocol endpoint, and an adapter that converts that protocol into the C2’s internal vehicle model.

- For a MAVLink drone, the minimum chain is:

MERMAID flowchart LR FC["Flight controller<br/>PX4 / ArduPilot"] AR["Air telemetry radio<br/>or IP modem"] GR["Ground radio / network endpoint"] GW["Edge gateway<br/>MAVLink adapter"] C2["C2 platform"] FC -->¦"UART / Ethernet<br/>MAVLink"¦ AR AR <-->¦"RF / LTE / mesh / Wi-Fi"¦ GR GR -->¦"USB / Ethernet / UDP"¦ GW GW <-->¦"Normalized telemetry<br/>authorized commands"¦ C2

# What hardware is actually required?

## Minimal MAVLink configuration

- On the drone:

• Flight controller such as a Pixhawk-class controller

• PX4 or ArduPilot

• GPS, IMU, compass and other flight sensors

• Air-side telemetry radio or IP modem

• Antenna

• Appropriate power supply

• Optional RC receiver for manual backup

- On the ground:

• Matching ground-side radio

• USB, serial or Ethernet connection

• Laptop or edge computer

• MAVLink adapter or ground-control application

• C2 application

- This can work without a companion computer. PX4 documents TELEM1 as the usual telemetry-radio connection and TELEM2 as the usual companion-computer connection. PX4 architecture

## Production-style configuration

- A more established autonomous/C2 product normally uses:

MERMAID flowchart LR subgraph UAV["On the UAV"] FC["Safety-critical<br/>flight controller"] MC["Linux mission/<br/>companion computer"] DL["IP data-link radio,<br/>LTE or mesh modem"] P["EO/IR or other payload"] FC <-->¦"MAVLink / DDS<br/>serial or Ethernet"¦ MC P <-->¦"Ethernet / USB / CAN"¦ MC MC <--> DL end subgraph Ground["On the ground"] GD["Ground radio/modem"] EC["Rugged edge computer"] UI["Operator laptop/tablet"] GD <--> EC EC <--> UI end DL <-->¦"Encrypted IP link"¦ GD

- The flight controller retains responsibility for stable flight and safety. The companion computer handles:

• C2 connectivity

• MAVLink routing

• Payload control

• Video encoding

• Computer vision

• Autonomous mission logic

• Store-and-forward operation

• Communications failover

• Vendor adapter containers

- PX4 explicitly recommends a companion computer when advanced processing is needed. It normally runs Linux and connects to the flight controller through serial or Ethernet using MAVLink or DDS. PX4 companion-computer architecture

# Examples from established products

## Auterion Skynode

- Skynode combines much of the vehicle integration stack:

• PX4 flight controller

• Linux mission computer

• Ethernet, CAN and serial interfaces

• Camera/video processing

• LTE and radio connectivity

• MAVLink interface

• Ground-control software

- Auterion Mission Control can discover a connected Skynode over USB networking, Wi-Fi or LTE. Their USB connection presents Skynode as a network device and uses DHCP before the ground application automatically discovers the vehicle. Auterion connection documentation

- This is an integrated commercial version of:

TEXT Pixhawk + companion computer + MAVLink router + modem + managed software

## Picogrid

- Picogrid illustrates the ground-side integration appliance model:

• Legion is its integration/data platform.

• Helios and related expeditionary nodes supply field power, communications and edge compute.

• Sensors and unmanned platforms connect locally to the node.

• The node translates and distributes their data to C2 systems.

- Picogrid describes Helios as bringing compute and communications to the tactical edge, while Legion provides the common interface. Picogrid architecture

## Anduril Lattice

- Lattice deployments use distributed tactical edge computers hosting the integration software near sensors, vehicles and command posts. The hardware-specific system talks to a local Lattice integration, and Lattice distributes the normalized state over its data mesh. Its public SDK exposes gRPC and REST interfaces. Lattice SDK

- The important pattern is that Lattice does not require every drone to speak the Lattice data model natively. An integration component translates the drone’s native protocol.

## Surtr ParallaxOS

- Surtr does not publish a detailed hardware specification. Reporting describes its current deliverable as an edge-compute unit with ParallaxOS installed. Sensors connect to that unit through standards or vendor APIs.

- Publicly confirmed Surtr interfaces are:

• SAPIENT

• Link 16

• Cursor-on-Target

• ATAK

• Custom vendor APIs

- MAVLink remains plausible but publicly unconfirmed.

- In practical terms, ParallaxOS probably requires:

• A computer capable of running its software at the site

• Ethernet or serial connectivity to sensors

• A receiver or modem for any wireless sensors

• Network access to the upstream C2

• Vendor adapters for each connected system

• Accurate sensor location, orientation and time configuration

- It does not eliminate the physical sensor receiver, drone modem or ground radio.

# The MAVLink handshake

- Importantly, pairing the radios is not part of MAVLink. The radio or IP network must first establish connectivity.

## Stage 1: physical and network connection

- The two endpoints first establish a transport:

• A paired telemetry-radio link

• Ethernet

• Wi-Fi

• LTE

• Tactical mesh

• Serial cable during bench testing

- The ground edge computer then receives MAVLink through something such as:

TEXT COM5 at 57600 or 921600 baud UDP 0.0.0.0:14550 TCP vehicle-address:5760 Ethernet interface

- A MAVLink router can forward the same vehicle stream to Sentinel, a local pilot station and a recorder simultaneously.

## Stage 2: vehicle discovery

- The autopilot broadcasts HEARTBEAT messages.

- A heartbeat supplies:

• System ID

• Component ID

• Vehicle type

• Autopilot family

• Flight mode

• Armed/disarmed flags

• Vehicle state

- On ordinary RF telemetry links, heartbeats are commonly transmitted around once per second. Missing several heartbeats indicates that the connection has been lost. MAVLink heartbeat specification

- Sentinel’s MAVLink adapter would see:

TEXT HEARTBEAT sysid: 14 compid: 1 type: QUADROTOR autopilot: PX4 mode: HOLD state: STANDBY

- It should then map sysid=14 to a durable Sentinel vehicle identity. A MAVLink system ID alone is not a sufficient enterprise identity because it can be reconfigured or duplicated.

## Stage 3: version and capability discovery

- The adapter requests AUTOPILOT_VERSION and component metadata.

- This establishes:

• MAVLink 1 or MAVLink 2

• Firmware version

• Hardware UID when exposed

• Supported mission protocol

• Supported positional-command format

• Parameter support

• File-transfer support

• Camera and gimbal components

• Vendor-specific dialects

- MAVLink provides a component-metadata protocol so a ground application can discover capabilities without hard-coding every vehicle. MAVLink component metadata

- Sentinel should compare the reported capability set with an allow-list:

TEXT Expected vehicle: sentinel-drone-07 Expected autopilot: PX4 Required: MAVLink 2 Required: mission-int Required: command-int Optional: camera, gimbal, FTP

## Stage 4: authentication and encryption

- MAVLink 2 can sign messages, allowing the receiver to verify that they came from someone with the shared secret.

- However, MAVLink signing does not encrypt the data. MAVLink security FAQ

- A production arrangement should therefore use:

• MAVLink 2 signing for message-level authenticity

• WireGuard, IPsec, radio encryption or another encrypted transport

• Device certificates or provisioned keys at the edge gateway

• An allow-list linking hardware identity to Sentinel vehicle identity

• Rotation and revocation of keys

- Sentinel should not mark a vehicle COMMANDABLE merely because it emits a heartbeat.

## Stage 5: time synchronization

- The adapter and vehicle exchange TIMESYNC messages to estimate clock offset and link delay.

- This is important for:

• Track association

• Video georegistration

• Sensor fusion

• Latency measurement

• Audit trails

• Replay

- Without synchronization, “the drone was here at 14:03:01” could mean different times to the vehicle, camera and C2.

## Stage 6: initial state synchronization

- The adapter waits for or requests:

• Position

• Attitude

• Velocity

• GPS quality

• Home position

• Battery state

• Estimator health

• Current mode

• Armed status

• Current mission identifier

• Camera and payload state

• Radio/link quality

- Only after these checks should the adapter transition through something like:

TEXT DISCOVERED → AUTHENTICATED → SYNCHRONIZING → TELEMETRY_READY → COMMANDABLE

- A drone may be visible but intentionally not commandable because:

• Firmware is unsupported

• Vehicle ID is unrecognized

• Signing failed

• Position estimate is unhealthy

• Time offset is unacceptable

• Another ground station owns control

• The safety pilot has not released authority

## Stage 7: command handshake

- Sentinel should send mission-level intent to its adapter:

JSON { "commandId": "cmd-9471", "vehicleId": "sentinel-drone-07", "type": "HOLD_POSITION", "expiresAt": "2026-07-28T10:15:05Z" }

- The adapter translates that into the appropriate MAVLink command addressed to the vehicle’s system and component IDs.

- MAVLink responds with COMMAND_ACK:

TEXT ACCEPTED IN_PROGRESS DENIED UNSUPPORTED TEMPORARILY_REJECTED FAILED

- If an acknowledgement is not received, the command protocol requires retry behavior. “Accepted” only means that the flight stack accepted the command, not necessarily that the physical action has completed. MAVLink command protocol

- For complete waypoint plans, the mission protocol performs a multi-message upload and finishes with MISSION_ACK. MAVLink mission protocol

# The sensor handshake is similar

- A radar or acoustic sensor may not use MAVLink. It typically connects to an edge computer through:

• Ethernet

• Serial

• CAN

• USB

• ONVIF/RTSP

• A vendor SDK

• SAPIENT

• ASTERIX

• Cursor-on-Target

- The edge adapter then performs:

MERMAID sequenceDiagram participant Sensor participant Adapter participant Edge participant C2 Sensor->>Adapter: Physical/IP connection Adapter->>Sensor: Query identity and capabilities Sensor-->>Adapter: Model, modalities, frames, task support Adapter->>Edge: Register source and sensor Edge-->>Adapter: Registration accepted Adapter->>Edge: Status and heartbeat Adapter->>Edge: Detections or local tracks Edge->>C2: Normalized observations C2->>Edge: Optional sensor task Edge->>Adapter: Look-at / mode / rate command Adapter->>Sensor: Vendor-native command Sensor-->>Adapter: Result Adapter-->>Edge: Command acknowledgement

- SAPIENT formalizes this with registration, status reports, detection definitions, task definitions, detection reports and task acknowledgements.

# Recommended Sentinel hardware architecture

- For the first physical-drone bench integration:

## On the drone

• PX4 or ArduPilot flight controller

• MAVLink-capable telemetry link

• Optional Linux companion computer

• Existing manual-control/failsafe path

• Camera only if needed for the demonstration

## At the edge

• Laptop or small industrial PC

• Ground telemetry-radio endpoint

• Ethernet/Wi-Fi/LTE as appropriate

• sentinel-mavlink-adapter

• MAVLink Router

• Recorder and health monitor

• Device identity/key store

## In Sentinel

TEXT MAVLink adapter ↓ Sentinel Edge Gateway ↓ Canonical VehicleState ↓ C2 world model ↓ Policy and operator authority ↓ VehicleCommand ↓ Edge Gateway ↓ MAVLink adapter ↓ COMMAND_ACK / MISSION_ACK

- The new adapter should be a separate process, just like the sensor simulator. Its southbound side speaks serial/UDP MAVLink, while its northbound side speaks Sentinel’s authenticated edge contract.

- The most important design rule is: the UI must never own the MAVLink connection. The edge adapter owns discovery, identity, synchronization, retries, command acknowledgements and failsafe behavior. The UI only sees a durable Sentinel vehicle and requests authorized mission actions.